URT Bevy URDF Viewer Onboarding Guide

Written by Frank Xia (please add to this document if you worked on this project to help others and future members)

# 1. Project Snapshot

RoboticsInRust is a Rust workspace for modelling and simulating a 6-DOF URT robotic arm. The repository contains multiple simulation approaches: a reusable DH-model library, a Kiss3D test viewer, a Bevy simulation, and the newer Bevy URDF viewer.

# 2. Fast Start

Recommended workflow from the repository root:

cd <your cloned file path>

cargo check -p bevy_sim_urdf

cargo run

It might take a while to build if this is your first time compiling this project. After compiling it once, you should be able to open the simulation window in just a few seconds.

Make sure you have enough storage on your laptop for this! This project might take 10+GB to run!

If you are tight with storage, you can do

cargo clean

to clean the files and get that storage back before you move on to something else.

If you are using Vscode as your code editor, you can install the extension “rust-analyzer” for checking definitions and implementations. It is a great tool as it builds the code at the start so you can view functions in the external libraries as well.

Useful runtime options:

| Option | Example | Effect |
| --- | --- | --- |
| --dry-run | cargo run -p bevy_sim_urdf -- --dry-run | Parse settings and URDF, then exit without opening a window. |
| --target-xyz | --target-xyz=-0.20,0.30,0.28 | Solve an initial task-space target using DLS IK. |
| --target-rpy | --target-rpy=0.0,0.0,1.57 | Constrain end-effector orientation as well as position. |
| --joints | --joints=-0.6,2.5,1.5,-1.2,-1.4,2.0 | Override startup joint angles. |
| --tri-cap | --tri-cap 25000 | Downsample each STL to keep rendering manageable. |

# 3. Controls in the Viewer

| Input | Mode | Behavior |
| --- | --- | --- |
| Q/A, W/S, E/D, R/F, T/G, Y/H | Joint mode | Increase/decrease each moving joint angle directly. |
| M | Task-space mode | Toggle DLS task-space IK on/off. |
| Arrow keys | Task-space mode | Move the red target in X/Y. |
| PageUp/PageDown (Fn + up/down arrow) | Task-space mode | Move the red target in Z. |
| Z/X, C/V, B/N | Task-space mode | Rotate target roll, pitch, and yaw; this enables orientation constraints. |
| Left/right mouse drag | Camera | Orbit around the robot. |
| Mouse wheel | Camera | Zoom in/out. |
| Space | Joint mode | Reset joint state values to zero. |

# 4. What You Are Looking At

The bevy_sim_urdf crate is a small application that loads a robot description from URDF, loads the visual meshes from STL files, creates a Bevy 3D scene, and lets the user move either individual joints or a task-space target.

| Mental model. The viewer has three layers: robot data from the URDF, Bevy entities that show that data on screen, and kinematics code that turns target movement into joint angles. |  |  |
| --- | --- | --- |
| Layer | What it means | Where to look first |  |
| Robot description | Define where everything is located. Links, joints, joint axes, origins, and mesh filenames parsed from the URDF file. | urdf.rs and model.rs |  |
| 3D viewer | Bevy entities, meshes, lights, camera, keyboard input, and debug joint axes. | scene.rs |  |
| Kinematics | The k crate chain plus our DLS inverse kinematics wrapper. | kinematics.rs |  |
| Startup settings | Command-line flags, default URDF path, initial joint angles, and optional startup target. | settings.rs |  |

# 5. Module Map for New Contributors

| File | Read this file when you need to... |
| --- | --- |
| main.rs | Understand app startup, Bevy plugin setup, and which systems run during Startup or Update. |
| settings.rs | Change command-line flags, default startup behaviour, default home pose, or startup IK target handling. |
| urdf.rs | Change how links, joints, colours, origins, axes, limits, or mesh references are parsed from URDF. |
| model.rs | Understand the plain structs that hold parsed robot data before it becomes Bevy entities. |
| mesh.rs | Change STL loading, triangle downsampling, normals, UVs, or mesh asset creation. |
| scene.rs | Change what appears in the Bevy window or how keyboard/mouse input affects the robot. |
| kinematics.rs | Change task-space IK, DLS parameters, target pose construction, or k crate integration. |
| constants.rs | Tune shared constants such as joint speed, target speed, home pose, and DLS damping values. |

# 6. Kinematics: The Whole Picture

Kinematics is the math that connects robot joint values to robot link poses. In this viewer, it has two directions: forward kinematics answers where the arm is for a given set of joint angles, and inverse kinematics answers what joint angle changes would move the end effector toward a desired target pose.

The important project idea is that the URDF is the single robot description, but it is used through two paths. Initially, our lightweight parser reads enough URDF data to draw the robot in Bevy. We then use the k library crate to read the same URDF to build a kinematic chain for FK, Jacobians, and IK.

| Question | Kinematics concept | Project code path |
| --- | --- | --- |
| Where is link6 right now? | Forward kinematics | Set joint positions on the k::Chain, call update_transforms, then read the target link world transform. |
| How should the joints change to move link6? | Inverse kinematics | Build a target pose, compute pose error, use the Jacobian, solve for a small joint increment, and iterate. |
| How does each joint affect the end effector? | Jacobian | k::jacobian(arm) returns the local linear map from joint increments to task-space motion. |
| How do we stay stable near singularities? | DLS IK | Add damping to the least-squares solve so tiny target movement does not produce extreme joint jumps. |

# 7. Forward Kinematics

Forward kinematics starts with joint values and computes the pose of every link. A pose means position plus orientation in 3D space. In this project, the usual FK question is: after applying the current six joint angles, where is link6?

Inputs: joint values, ordered by the movable joints from the URDF chain.

Output: world transforms for links, especially the end-effector link named link6.

Where to look: create_kinematics_state and link_position_from_joint_values in kinematics.rs.

FK is used at startup to place the target marker at the current end-effector position when no explicit target is provided. It is also the reference point for IK, because IK needs to know the current end-effector pose before it can reduce the error to the target.

# 8. Inverse Kinematics Runtime Flow

Inverse kinematics goes the other way: it starts with a desired end-effector pose (red cube) and searches for joint values that produce it. Our implementation is iterative, so it does not solve the entire movement in one large jump. Each update asks for a small joint correction, applies it, recomputes the current pose, and repeats until the error is small enough or the solver gives up.

The user moves the red target marker in task-space mode using the keyboard.

drive_task_space_target updates the desired XYZ position and, if enabled, desired RPY orientation.

solve_task_space_ik_values copies the current joint map into the kinematic chain.

solve_task_space_ik builds a target transform for link6 and creates a serial chain ending at that link.

solve_damped_least_squares_ik repeatedly computes pose error, builds a Jacobian, solves for dq, and applies the joint increment.

If the solve converges, the returned joint map becomes the viewer state and Bevy redraws the robot links.

| Function | Role in the IK path |
| --- | --- |
| drive_task_space_target | Runtime Bevy system. Reads input, moves the target marker, and calls IK when task-space mode is enabled. |
| solve_task_space_ik_values | Bridge from Bevy joint state to k crate joint positions; returns an updated HashMap of joint angles. |
| solve_task_space_ik | Builds the desired link6 transform and chooses whether orientation is constrained. |
| solve_damped_least_squares_ik | The actual numerical IK loop using the Jacobian damped least-squares update. |

# 9. Jacobian-Based IK

This code is Jacobian-based IK. The Jacobian is a matrix that describes the local relationship between small joint changes and small end-effector motion. In compact form:

dx = J dq

Here, dx is the task-space error we want to reduce, J is the Jacobian, and dq is the joint angle increment in radians. dq is not a joint velocity command. It is the small angle change that the solver wants to add to the current joint values for this iteration.

We used the k library to help us set up Jacobian

let full_jacobian = k::jacobian(arm);

Refer to the k library and the robotics workshop for more information.

| Term | Meaning |
| --- | --- |
| Task space | The end-effector space the user cares about: XYZ position and optionally roll, pitch, yaw orientation. |
| Joint space | The six joint angles that define the robot pose. |
| Jacobian column | How one joint affects the end-effector when that joint changes slightly. |
| Jacobian row | One task-space direction being controlled, such as X, Y, Z, roll, pitch, or yaw. |

# 10. What Is DLS And What Is It Used For

DLS stands for Damped Least Squares. It is not a separate control mode; it is the numerical method used inside our inverse kinematics solver. Its job is to convert the current task-space error into a stable joint angle increment (refer to the robotics workshop).

dq = J^T (J J^T + lambda^2 I)^-1 dx

A plain Jacobian inverse can become unstable near singularities, where the arm loses the ability to move cleanly in some direction. Near those poses, a tiny target movement can produce a very large joint update, which can cause glitches in the simulation or very large spikes that could burn the motors in real life. DLS adds lambda^2 I before solving, which makes the matrix better conditioned and keeps the update more predictable.

Our implementation also adapts damping based on the smallest singular value of the Jacobian. When the Jacobian looks close to singular, damping increases from DLS_BASE_DAMPING toward DLS_BASE_DAMPING + DLS_SINGULARITY_DAMPING. Each joint increment is then scaled and clamped to DLS_MAX_JOINT_STEP so one frame cannot apply a huge jump.

| Constant | Current role |
| --- | --- |
| DLS_ALLOWABLE_TARGET_DISTANCE | Position error threshold for declaring convergence. |
| DLS_ALLOWABLE_TARGET_ANGLE | Orientation error threshold when orientation is constrained. |
| DLS_BASE_DAMPING | Always-present damping in the solve. |
| DLS_SINGULARITY_THRESHOLD | Smallest singular value threshold where extra damping starts to matter. |
| DLS_MAX_TRIES | Maximum number of internal IK iterations before the solver reports non-convergence. |

## Position-only vs position-and-orientation IK

The solver can run with either three task-space rows or six task-space rows. If no target RPY is active, it only constrains XYZ position. If the user rotates the target marker or passes --target-rpy, it also constrains orientation.

Position-only IK is usually easier to solve because the wrist orientation is free to land wherever it naturally lands.

Position-and-orientation IK is stricter because the same six joints must satisfy both XYZ and roll/pitch/yaw error.

In the code, this is controlled by target_rpy: Option<Vec3> and the operational_space boolean array in kinematics.rs.

# 11. URDF: What It Is and How We Use It

As previously mentioned, URDF stands for Unified Robot Description Format. It is an XML format commonly used in ROS-style robotics projects to describe a robot as a tree of links connected by joints. A URDF file is not motor firmware and not a full physics simulator by itself; it is the structural model that tells software what the robot is made of and how the pieces are connected.

Although we are not using ROS or ROS2, we can still use URDF, think of it as a way to describe robotics.

A typical URDF can describe link names, joint names, parent-child relationships, fixed and moving joints, joint axes, joint origins, joint limits, visual meshes, collision meshes, inertial properties, colours, and mesh scales. This project currently uses the parts needed for visualisation and kinematics.

| URDF element | What it means | How this viewer uses it |
| --- | --- | --- |
| link | A rigid body in the robot tree. | Creates renderable robot parts and gives k a named link to solve toward. |
| joint | The transform and motion relationship between parent and child links. | Defines axes, origins, parent-child hierarchy, and which joints are movable. |
| origin xyz/rpy | Position and orientation offset for a visual or joint. | Places meshes and joints correctly in the Bevy scene and kinematic chain. |
| axis xyz | The axis a revolute or continuous joint rotates around. | Determines how a key press or IK joint increment changes the robot pose. |
| mesh filename | Path to an STL or other mesh asset. | Loads the visible geometry for each link. |
| limit lower/upper | Allowed range for limited revolute joints. | Used only for revolute joints; continuous joints are treated as unlimited. |

How to get the URDF file and how to update the meshes:

An URDF file can be handwritten or generated by your CAD software. Luckily, OnShape does support exporting assemblies as URDF files. Right-click on your assembly’s name at the bottom, select export, choose the format as URDF and change the resolution if you wish.

It will export a zip file that contains both the meshes, the .urdf file, and probably a launch file. We do not need the launch file as it is built for ROS, so just copy the mesh files into the asset folder. The code is not path dependent, so the new meshes will be rendered if you build it again.

However, you might notice that the exported URDF file contains a lot of redundant information that we do not need, and it can mess up the simulation. This is because Onshape does not treat rigid bodies as one link, and so it will produce all the objects and possibly treat them as individual links. The way to solve this is to:

- Make sure all mate connectors are fastened mates, only use revolute mates for the motor joints.

- Regroup the objects into 6 links, the revolute joints will now become hints when grouping these objects. (or you can use the file collapse_fixed_urdf.py, but it might not be usable)

# 12. Continuous Joints and Multi-Turn Motors

At the moment, the ZeroErr motors we have been using are multi-turn motors, meaning that they can remember their positions even when the power is off and are able to turn more than 360 degrees. The new motors that we are getting might be single-turn motors, which means they can only rotate one revolution.

The arm joints are named revolute_1 through revolute_6, but the URDF joint type is continuous. That is the right representation for multi-turn motors that can rotate through more than one full revolution.

<joint name="revolute_1" type="continuous">

...

<joint name="revolute_6" type="continuous">

- If in the future we decide to go with the single-turn motors, change these joints to type="revolute", then lower and upper limits become meaningful and should be checked during debugging.